ASSIGNMENT 5

1. Key Features of Wi-Fi 6, 6E, and 7 + Comparison with Wi-Fi 5

| Feature | Wi-Fi 5 | Wi-Fi 6 | Wi-Fi 6E | Wi-Fi 7 |
| --- | --- | --- | --- | --- |
| Standard | 802.11ac | 802.11ax | 802.11ax | 802.11be |
| Bands | 5 GHz | 2.4 + 5 GHz | + 6 GHz | 2.4 + 5 + 6 |
| OFDMA | ❌ | ✅ | ✅ | ✅ |
| MU-MIMO | Downlink | UL + DL | UL + DL | Advanced |
| Modulation | 256-QAM | 1024-QAM | 1024-QAM | 4096-QAM |
| Channel Width | 160 MHz | 160 MHz | 160 MHz | 320 MHz |

2. Role of OFDMA in Wi-Fi 6 & How it Improves Efficiency

OFDMA = Multi-user version of OFDM

- Channel is divided into subcarriers

- Subcarriers grouped into Resource Units (RUs)

- Each RU → assigned to different user

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| Users | One at a time | Multiple |
| Efficiency | Medium | High |
| Latency | Higher | Lower |

3. Benefits of Target Wake Time (TWT) in Wi-Fi 6 for IoT Devices

TWT allows a device and Access Point (AP) to agree on specific wake-up times.

| Feature | Without TWT | With TWT |
| --- | --- | --- |
| Power usage | High | Low |
| Wake time | Random | Scheduled |
| Efficiency | Low | High |
| Battery life | Short | Long |

4. Significance of the 6 GHz Band in Wi-Fi 6E

The 6 GHz band, introduced in Wi-Fi 6E, provides additional spectrum for wireless communication, offering more channels, reduced interference, higher data rates, and improved performance compared to traditional 2.4 GHz and 5 GHz bands.

Range - 5.925 GHz – 7.125 GHz

| Feature | 2.4 GHz | 5 GHz | 6 GHz |
| --- | --- | --- | --- |
| Range | Long | Medium | Short |
| Speed | Low | High | Very High |
| Interference | High | Medium | Very Low |
| Channels | Few | More | Many |

5. Compare Wi-Fi 6 and Wi-Fi 6E (Range, Bandwidth, Interference)

| Feature | 2.4 GHz | 5 GHz | 6 GHz |
| --- | --- | --- | --- |
| Range | Long | Medium | Short |
| Speed | Low | High | Very High |
| Interference | High | Medium | Very Low |
| Channels | Few | More | Many |

6. Major Innovations in Wi-Fi 7 (802.11be)

320 MHz Channel Width - Double Wi-Fi 6 (160 MHz → 320 MHz)

4096-QAM Modulation - Higher bits per symbol

Multi-Link Operation (MLO) - Uses multiple bands simultaneously

Enhanced MU-MIMO - More spatial streams, Better multi-user support

Multi-RU Allocation (OFDMA Improvement) - One user can use multiple resource units

Ultra-Low Latency

Preamble Puncturing (Flexible Channels) - Avoid interference in part of channel

7. Multi-Link Operation (MLO) + Impact on Throughput & Latency

Multi-Link Operation (MLO) in Wi-Fi 7 allows a device to use multiple frequency bands (2.4 GHz, 5 GHz, and 6 GHz) simultaneously, improving throughput, reducing latency, and enhancing reliability

| Feature | Without MLO | With MLO |
| --- | --- | --- |
| Bands used | One | Multiple |
| Throughput | Limited | High |
| Latency | Higher | Lower |
| Reliability | Medium | High |

8. Purpose of 802.11k and 802.11v + How they aid roaming

802.11k provides radio resource measurements to help clients identify better access points, while 802.11v enables network-assisted roaming by guiding clients to optimal access points. Together, they improve roaming efficiency and reduce connection delays.

When user moves → device must switch AP

Without optimization:

Slow roaming

Packet loss

Poor experience

| Feature | 802.11k | 802.11v |
| --- | --- | --- |
| Function | Provides info | Guides decision |
| Role | Measurement | Control |
| Helps in | Faster scanning | Better roaming decision |

9. Fast BSS Transition (802.11r) + Benefits

Step 1: Initial Connection

Step 2: Key Sharing

Step 3: Fast Roaming

| Feature | Without 802.11r | With 802.11r |
| --- | --- | --- |
| Authentication | Full | Fast |
| Delay | High | Very low |
| Experience | Poor | Smooth |

10. How 802.11k, 802.11v, and 802.11r Work Together for Seamless Roaming

Step 1: 802.11k → Client gets AP list

Step 2: 802.11v → AP suggests best AP

Step 3: 802.11r → Fast secure transition

Seamless roaming achieved

Benefits :

Seamless roaming

Zero/low packet loss

No call drops

Low latency

Better user experience