ASSIGNMENT 1

1. In which OSI layer does Wi-Fi standard/protocol fit?

Layer 1 – Physical Layer - data sent over Wi-Fi, it converts it into radio signals

Layer 2 – Data Link Layer - Controls who can send data and when using CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

2. Wi-Fi devices you use + their wireless properties + Wi-Fi generation mapping

Laptop

SSID - KAVIN SKYLINK -5G

Protocol - Wi-Fi 5 (802.11ac)

Phone

SSID – KAVIN SKYLINK -2.4G

Protocol – Wi-Fi 4 (802.11n)

3. BSS

1 Access Point (AP)

Multiple client devices

Example. Home Wi-Fi

ESS

Group of multiple BSS networks connected together

Allows seamless roaming

Example. College Wi-Fi (Lab, Classroom all have same Wi-Fi)

4. Basic functionalities of a Wi-Fi Access Point

Bridge between Wireless devices (phones, laptops) Wired network (router / internet)

Data → Radio waves (to send)

Radio waves → Data (to receive)

Verifies users before allowing access (Password)

Controls who can transmit data and when, CSMA/CA (Collision Avoidance)

Helps devices switch between APs smoothly

Sends out the Wi-Fi network name (SSID)

5. Bridge Mode

Used to connect two network segments and make them behave like a single network.

Works at Layer 2 (Data Link Layer)

Wired/Wireless

Repeater Mode

Used to extend Wi-Fi range by repeating the existing signal.

Reduced speed

Wireless only

6. Differences between 802.11a and 802.11b

802.11a

Frequency: 5 GHz

Maximum Speed: 54 Mbps

Modulation: OFDM - Orthogonal Frequency Division Multiplexing

Range: Shorter
	802.11b

Frequency: 2.4 GHz

Maximum Speed: 11 Mbps

Modulation: DSSS - Direct Sequence Spread Spectrum

Range: Longer

7. 5GHz

2.4GHz

8. IEE - Institute of Electrical and Electronics Engineers

International organization that creates technical standards

Frequency bands

Modulation (OFDM, DSSS)

Data rates

Protocol rules

WFA - Wi-Fi Alliance

Organization that ensures devices work together properly

Ensures compatibility between brands

Gives simple names like:

Wi-Fi 4

Wi-Fi 5

Wi-Fi 6

9. Backhaul

The link between your router and the internet

Fiber

DSL

Cable

Cellular

10. Wi-Fi topologies

Infrastructure Topology - Devices connect through a central Access Point (AP)

Home Wi-Fi, Offices, Colleges

Ad-hoc Topology (IBSS) - Devices connect directly to each other (no Access Point)

Temporary file sharing, Small group communication

Mesh Topology - Multiple access points connected together forming a mesh network

Large campuses, Smart homes

Point-to-Point (P2P) Topology - Direct connection between two locations using wireless link

Connecting two buildings

Point-to-Multipoint (P2MP) - One central AP connects to multiple remote devices

Wireless ISP (WISP), Rural internet distribution