ASSIGNMENT 3

1. Different 802.11 PHY layer standards + comparison

Defines how data is transmitted over the air using different frequencies, modulation techniques, and data rates

| Standard | Band | Modulation | Max Speed | Key Feature |
| --- | --- | --- | --- | --- |
| 802.11b | 2.4 GHz | DSSS | 11 Mbps | Long range |
| 802.11a | 5 GHz | OFDM | 54 Mbps | Low interference |
| 802.11g | 2.4 GHz | OFDM | 54 Mbps | Balanced |
| 802.11n | 2.4/5 | OFDM | 600 Mbps | MIMO |
| 802.11ac | 5 GHz | OFDM | ~3.5 Gbps | MU-MIMO |
| 802.11ax | 2.4/5/6 | OFDMA | ~9.6 Gbps | High efficiency |

2. What are DSSS and FHSS? How do they work?

DSSS (Direct Sequence Spread Spectrum)

Each data bit is converted into multiple bits (chips) using a spreading code.

Data bit (1 or 0)

Multiply with a pseudo-random code

Transmit across wider band

FHSS (Frequency Hopping Spread Spectrum)

Signal jumps between different frequencies rapidly during transmission.

Data is divided into small chunks

Transmitted on different frequencies

Frequency changes quickly (hopping)

3. How do modulation schemes work in PHY layer? Compare different schemes

Modulation schemes in the PHY layer convert digital data into radio signals by varying signal 	properties such as amplitude, frequency, or phase

Modulation = converting digital data (0s & 1s) into signals for transmission over air

| Scheme | Bits/Symbol | Speed | Reliability |
| --- | --- | --- | --- |
| BPSK | 1 | Low | High |
| QPSK | 2 | Medium | Good |
| 16-QAM | 4 | Higher | Moderate |
| 64-QAM | 6 | High | Lower |
| 256-QAM | 8 | Very High | Needs strong signal |
| 1024-QAM | 10 | Extremely High | Very sensitive |

4.  What is the significance of OFDM in WLAN? How does it improve performance?

OFDM splits data into multiple parallel streams and sends them over different sub-frequencies (subcarriers).

- Data is divided into smaller chunks

- Each chunk is assigned to a subcarrier

- All subcarriers transmit at the same time

Higher Data Rate

Better Resistance to Interference

Handles Multipath Fading

Efficient Bandwidth Usage

Reduced ISI (Inter-Symbol Interference)

5. How are frequency bands divided for Wi-Fi? Explain different bands and channels

| Feature | 2.4 GHz | 5 GHz | 6 GHz |
| --- | --- | --- | --- |
| Range | Long | Medium | Short |
| Speed | Low | High | Very High |
| Interference | High | Low | Very Low |
| Channels | Few | Many | Very Many |

A channel is a smaller frequency range inside a band used for communication.

Channel 1 → 2.412 GHz 
		Channel 6 → 2.437 GHz 
		Channel 11 → 2.462 GHz

6. Role of Guard Interval (GI) in WLAN + Short GI

Guard Interval is a small time gap between symbols in wireless transmission.

| Feature | Normal GI | Short GI |
| --- | --- | --- |
| Reliability | High | Slightly lower |
| Speed | Lower | Higher |
| Use Case | Long distance | Good signal conditions |

7. Structure of an 802.11 PHY Layer Frame

A PHY frame is the actual signal format transmitted over the air.

| Preamble | Header (PLCP Header) | Payload (Data) |

| Component | Function |
| --- | --- |
| Preamble | Synchronization |
| Header | Transmission info |
| Payload | Actual data |

8. Difference between OFDM and OFDMA

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| Users | Single user | Multiple users |
| Channel Usage | Entire channel per user | Shared channel |
| Efficiency | Lower (many users) | High |
| Latency | Higher | Lower |
| Use Case | Simple networks | Dense networks |

9. Difference between MIMO and MU-MIMO

| Feature | MIMO | MU-MIMO |
| --- | --- | --- |
| Users | Single user | Multiple users |
| Streams | Multiple streams to one | Streams to many |
| Efficiency | Medium | High |
| Speed | High (single user) | Balanced (many users) |

10. What are PPDU, PLCP, and PMD in the PHY layer?

| Component | Role | Function |
| --- | --- | --- |
| PMD | Transmission | Sends signals |
| PLCP | Conversion | Prepares data |
| PPDU | Frame | Final transmitted data |

11. Types of PPDU & frame formats across Wi-Fi generations

| PPDU Type | Standard | Key Feature |
| --- | --- | --- |
| Legacy | a/b/g | Basic transmission |
| HT | n | MIMO |
| VHT | ac | MU-MIMO |
| HE | ax | OFDMA |

12. How is the data rate calculated in Wi-Fi?

The data rate in Wi-Fi is calculated based on channel bandwidth, modulation scheme (bits per symbol), coding rate, number of spatial streams, and guard interval.

Nsc = Number of Subcarriers

Nbpsc = Bits per Subcarrier

R = Coding Rate

Nss = Spatial Streams

Tsym = Symbol Duration