ASSIGNMENT 6

1. Pillars of Wi-Fi Security

| Pillar | Purpose |
| --- | --- |
| Authentication | Verify identity |
| Encryption | Protect data |
| Integrity | Ensure correctness |

2. Difference Between Authentication and Encryption in Wi-Fi Security

| Feature | Authentication | Encryption |
| --- | --- | --- |
| Purpose | Verify identity | Protect data |
| When used | Before connection | During data transfer |
| Focus | Access control | Data privacy |
| Example | Password login | AES encryption |

3. Differences Between WEP, WPA, WPA2, and WPA3

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Encryption | RC4 | TKIP | AES | AES (Enhanced) |
| Security | Very weak | Weak | Strong | Very strong |
| Key Management | Static | Dynamic | Dynamic | Advanced (SAE) |
| Status | Obsolete | Deprecated | Widely used | Latest |

4. Why is WEP considered insecure compared to WPA2/WPA3?

WEP is considered insecure because it uses weak encryption (RC4), static keys, and poor key management, making it vulnerable to attacks.

| Feature | WEP | WPA2/WPA3 |
| --- | --- | --- |
| Encryption | RC4 (weak) | AES (strong) |
| Key Type | Static | Dynamic |
| Security | Very low | High |
| Attack Resistance | Poor | Strong |

5. Why was WPA2 introduced?

WPA2 was introduced to replace WPA and WEP by providing stronger security through AES encryption, improved key management, and better protection against known vulnerabilities.

| Feature | WPA | WPA2 |
| --- | --- | --- |
| Encryption | TKIP | AES |
| Security | Moderate | Strong |
| Standard | Temporary | IEEE standard |
| Reliability | Medium | High |

6. Role of Pairwise Master Key (PMK) in the 4-Way Handshake

The Pairwise Master Key (PMK) is the base key used in the 4-way handshake to derive the Pairwise Transient Key (PTK), which is used for encrypting communication between the client and the access point.

PMK → used to generate → PTK → used for encryption

During handshake:

- AP and Client already share PMK

- Exchange:

ANonce

SNonce

- Both generate:
	PTK using PMK + nonces

7. How does the 4-Way Handshake ensure mutual authentication?

The 4-way handshake ensures mutual authentication by verifying that both the client and the access point possess the same Pairwise Master Key (PMK) without transmitting it, using nonce values and message integrity checks.

Step 1: AP → Client (ANonce)

Step 2: Client → AP

Step 3: AP → Client

Step 4: Client → AP

MIC = Message Integrity Code

Verifies:

Message is not altered

Sender has correct key

8. What happens if we enter a wrong passphrase during the 4-way handshake?

If a wrong passphrase is entered, the client and access point generate different PMKs, resulting in mismatched PTKs during the 4-way handshake. This causes message integrity check (MIC) failure, and the connection is rejected.

Step 1: PMK Generation

Step 2: PTK Generation

Step 3: MIC Verification Failure

Step 4: Handshake Failure

Wrong Password → Wrong PMK → Wrong PTK → MIC mismatch → Connection failed

9. What problem does 802.1X solve in a network?

802.1X solves the problem of unauthorized network access by providing centralized authentication for users and devices, ensuring that only verified users can connect to the network.

In normal Wi-Fi (PSK):

Same password shared by everyone

If password leaked → anyone can connect

1. Unauthorized Access

Only authenticated users allowed

2. Shared Password Problem

Each user has Unique credentials

3. Lack of Central Control

Uses authentication server

4. No User Tracking

Now, Each user is identified

Flow:

- Client requests access

- AP forwards to server

- Server verifies credentials

- Access granted/denied

| Feature | Without 802.1X | With 802.1X |
| --- | --- | --- |
| Authentication | Shared password | Individual login |
| Security | Low | High |
| Control | Poor | Centralized |
| User tracking | No | Yes |

10. How does 802.1X enhance security over wireless networks?

802.1X enhances wireless security by providing centralized authentication, dynamic key generation, and strong encryption through protocols like EAP, ensuring that only authorized users can access the network securely.

Individual User Authentication

Centralized Authentication

Dynamic Key Generation

Strong Encryption

Mutual Authentication

Better Access Control

Logging & Monitoring

| Feature | PSK | 802.1X |
| --- | --- | --- |
| Authentication | Shared | Individual |
| Security | Medium | High |
| Key Type | Static | Dynamic |
| Control | Limited | Centralized |