ASSIGNMENT 4

1. Significance of MAC layer + Position in OSI model

Layer 3 → Network (IP)

Layer 2 → Data Link (MAC + LLC)

Layer 1 → Physical

Medium Access Control

Frame Delivery

Addressing

Error Detection

Retransmission

Fragmentation

Power Management

Security Handling

2. 802.11 MAC Frame Format + Field Explanation

| Frame Control | Duration | Address 1 | Address 2 | Address 3 | Sequence Control | Address 4 (optional) | Payload | FCS |

| Field | Function |
| --- | --- |
| Frame Control | Frame type & flags |
| Duration | Channel reservation |
| Address fields | Identify devices |
| Sequence Control | Order & fragments |
| Payload | Actual data |
| FCS | Error detection |

3. MAC Layer Functionalities (Management, Control, Data Planes)
	Management handles network setup and maintenance, Control manages access to the medium, and Data is responsible for actual data transmission.

| Plane | Purpose | Functions |
| --- | --- | --- |
| Management | Setup network | Scan, auth, associate |
| Control | Medium access | RTS/CTS, ACK |
| Data | Data transfer | Frame transmission |

4. Scanning Process in Wi-Fi + Types

Scanning is the process where a client device searches for available Access Points (APs).

Start → Scan Channels → Detect AP → Collect Info → Choose Best AP

| Feature | Passive | Active |
| --- | --- | --- |
| Method | Listen | Send request |
| Speed | Slow | Fast |
| Power | Low | Higher |
| Traffic | Low | Higher |

5. Client Association Process in Wi-Fi

Association is the process where a client officially joins an Access Point (AP) and becomes part of the network.

Scan → Authenticate → Association Request → Association Response → Connected

6. EAPOL 4-Way Handshake + Keys Explanation

EAPOL = Extensible Authentication Protocol over LAN

Purpose of 4-Way Handshake

Generate encryption keys

Verify both client & AP

Protect data transmission

PMK (Pairwise Master Key)

PTK (Pairwise Transient Key)

GTK (Group Temporal Key)

Handshake Flow

1. AP → Client : ANonce

2. Client → AP : SNonce + MIC

3. AP → Client : GTK + MIC

4. Client → AP : ACK

7. Power Saving Mechanisms in MAC Layer

The MAC layer provides power-saving mechanisms that allow wireless devices to conserve battery by switching between active and sleep modes.

Legacy Power Save Mode (PSM)

Client informs AP → going to sleep

AP buffers data

AP sends Beacon with TIM (Traffic Indication Map)

Client wakes up → checks data

Requests buffered data

U-APSD (Unscheduled Automatic Power Save Delivery)

Client sends trigger frame

AP sends all buffered data immediately

TWT (Target Wake Time) – Wi-Fi 6

Device sleeps for long time

Wakes up only at scheduled time

8. Medium Access Control Methodologies in WLAN

Medium Access Control methodologies define how multiple devices share the wireless medium.

CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

Device checks if channel is free

If busy → waits

If free → waits random backoff time

Transmits data

DCF (Distributed Coordination Function)

PCF (Point Coordination Function)

AP polls devices

Gives permission to transmit

EDCA (Enhanced Distributed Channel Access)

Prioritize traffic

| Method | Control Type | Feature |
| --- | --- | --- |
| CSMA/CA | Basic | Avoid collisions |
| DCF | Distributed | Default method |
| PCF | Centralized | AP controls |
| EDCA | Priority-based | QoS support |

9. Block ACK Mechanism & Advantages

Block Acknowledgment (Block ACK) is a mechanism in WLAN where multiple data frames are acknowledged using a single acknowledgment frame, improving efficiency and reducing overhead.

How Block ACK Works

Sender transmits multiple frames (burst)

Receiver collects frames

Receiver sends one Block ACK

| Feature | Normal ACK | Block ACK |
| --- | --- | --- |
| ACK Count | One per frame | One for many frames |
| Efficiency | Low | High |
| Overhead | High | Low |
| Speed | Lower | Higher |

10. A-MSDU, A-MPDU and A-MSDU in A-MPDU

A-MSDU and A-MPDU are frame aggregation techniques used in WLAN to improve efficiency.

A-MSDU (Aggregate MSDU) - Aggregate MAC Service Data Unit

| MSDU1 | MSDU2 | MSDU3 | → Single MPDU |

A-MPDU (Aggregate MPDU) - Aggregate MAC Protocol Data Unit

| MPDU1 | MPDU2 | MPDU3 | → One transmission |

A-MSDU inside A-MPDU

| MPDU1 (MSDU1+MSDU2) | MPDU2 (MSDU3+MSDU4) |

| Feature | A-MSDU | A-MPDU |
| --- | --- | --- |
| Aggregation Level | MSDU | MPDU |
| Error Handling | Poor | Good |
| Efficiency | High | High |
| Reliability | Low | High |